Lab 4 Building the functional blocks of the reduced set RISC-V datapath

Objective

In this lab, we will describe and simulate three components of the reduced set RISC-V architecture.

Hardware description of the Functional blocks

The three functional blocks that this laboratory period will be focusing on are the data memory, the register file, and the ALU.

Data memory

The data memory is created out of Random Access Memory (RAM), we will keep the size of the memory and the precision of each location in the memory generic. You can always read the location of the memory that is addressed by the address lines, but you can only write a specific location if the write enable is high on a rising clock edge.

The entity description of this component and its signal definitions in vhdl are as follows:

library IEEE; use IEEE.STD_LOGIC_1164.ALL;

use IEEE.NUMERIC_STD_UNSIGNED.ALL;

entity ram_array is

generic(N: integer := 6; M: integer := 32);

port(clk,

we: in std_logic;

adr: in STD_LOGIC_VECTOR(N-1 downto 0);

din: in STD_LOGIC_VECTOR(M-1 downto 0);

dout: out STD_LOGIC_VECTOR(M-1 downto 0));

end;

architecture synth of ram_array is

type mem_array is array ((2**N-1) downto 0) of STD_LOGIC_VECTOR (M-1 downto 0);

signal mem: mem_array;

Register File

The register file is fixed to 32 registers with each register having a precision of 32 bits. You can always read two different locations in that register file and only write to a third location if write enable is high on a rising edge. Also register 0 is hardwired to zero.

The entity description of this component and its signal definitions in vhdl are as follows:

library IEEE;

use IEEE.STD_LOGIC_1164.all;

use IEEE.NUMERIC_STD_UNSIGNED.all;

entity regfile is

port(clk: in STD_LOGIC;

we3: in STD_LOGIC;

a1, a2, a3: in STD_LOGIC_VECTOR(4 downto 0);

wd3: in STD_LOGIC_VECTOR(31 downto 0);

rd1, rd2: out STD_LOGIC_VECTOR(31 downto 0));

end;

architecture behave of regfile is

type ramtype is array (31 downto 0) of STD_LOGIC_VECTOR

(31 downto 0);

signal mem: ramtype;

ALU

We are not going to build this component; we will just simulate it. The vhdl description is as follows:

library IEEE; use IEEE.STD_LOGIC_1164.ALL;

use IEEE.NUMERIC_STD_UNSIGNED.ALL;

entity alu is

port(a, b:       in     STD_LOGIC_VECTOR(31 downto 0);

ALUControl: in     STD_LOGIC_VECTOR(2  downto 0);

ALUResultout:  out STD_LOGIC_VECTOR(31 downto 0);

Zero:       out    STD_LOGIC);

end;

architecture behave of alu is

signal ALUResult, condinvb, sum: STD_LOGIC_VECTOR(31 downto 0);

begin

condinvb <= not b when Alucontrol(0)='1' else b;

sum <= a + condinvb + Alucontrol(0);

process(a,b,ALUControl,sum) begin

case Alucontrol is

when "000" =>  ALUResult <= sum;

when "001" =>  ALUResult <= sum;

when "010" =>  ALUResult <= a and b;

when "011" =>  ALUResult <= a or b;

when "101" =>  ALUResult <= (0 => sum(31), others => '0');

when others => ALUResult <= (others => 'X');

end case;

end process;

ALUResultout <= ALUResult;

Zero <= '1' when ALUResult = X"00000000" else '0';

end;

Simulation

Each component will be simulated to test its functionality. Hints on simulation process/data will be given in the laboratory period and the supporting document on simulation in vivado.

Deliverables

This is a formal laboratory report. Your report should contain the following:

Introduction (15 points): specify the importance of these three different components in building a RISC-V architecture and give a short description of each one of them.

Procedure (35 points): Include the hardware description of all three components with comments and descriptions of the different steps done in the laboratory for simulation.

Results (35 points): Include the simulation results with comments.

Conclusion (15 points): Reflect on the laboratory and specify what questions you still have that are still open and not answered.